SysTec Smart Thermostat — Architecture & Cloud Integration Report

Executive Summary

We prototyped a working thermostat on Raspberry Pi using: an AHT20 temperature/-humidity sensor over I²C, a 16×2 HD44780 LCD (4-bit parallel), three momentary buttons (mode, set-point up, set-point down), two LEDs (red = heat, blue = cool) driven with PWM for fade/solid behavior, and a UART telemetry stream. This report evaluates three production-class hardware architectures—Raspberry Pi (Linux SBC), Microchip PIC32MZ-W1 (WFI32E01), and NXP i.MX RT + IW416 Wi-Fi—against SysTec’s requirements: peripheral support, Wi-Fi connectivity, and adequate Flash/RAM. I recommend Microchip’s PIC32MZ-W1 (WFI32E01) as the best-fit production platform due to its integrated 802.11 b/g/n Wi-Fi, ample on-chip Flash/RAM, rich MCU-class peripherals (GPIO/PWM/I²C/UART), lower power, and simpler certification path than a Linux SBC or a discrete MCU + Wi-Fi combo. (Raspberry Pi Forums)

Prototype Recap (What we built)

- Sensors/Actuators: AHT20 on I²C; two LEDs (PWM fade for “active” heat/cool, solid at set-point); three buttons; 16×2 LCD (RS/E/D4–D7); UART CSV output every 30 s.

- State machine: OFF → HEAT → COOL → (wrap) via Mode button; set-point ±0.5 °F via Up/Down; LED logic and LCD text driven by current state and temperature delta.

- Why it matters: The production target must, at minimum, expose GPIO with PWM, I²C, and UART; drive a simple character LCD; and host reliable Wi-Fi with enough memory for networking, OTA, and security.

Options Considered

A. Raspberry Pi (e.g., Pi 3-series / Zero W)

- What it offers: 40-pin header with I²C, multiple GPIO (software PWM), UART; built-in Wi-Fi/BLE (Broadcom combo); 512 MB–1 GB RAM; Linux software stack. (Raspberry Pi)

- Pros: Rapid prototyping; rich Linux ecosystem; easy Python re-use; TLS stacks and cloud SDKs are mature.

- Cons: Higher power; SD-card reliability concerns; full Linux adds attack surface and maintenance; certification & BOM cost can be higher than an MCU-class module for a mass-market thermostat.

B. Microchip PIC32MZ-W1 (WFI32E01 Module) — Recommended

- What it offers: Single module with PIC32MZ MCU + 802.11 b/g/n Wi-Fi; up to 2 MB Flash and 512 KB RAM; hardware peripherals (I²C, UARTs, PWM timers, GPIO); production-oriented SDKs (Harmony, FreeRTOS). (w11stop.com)

- Pros: Integrated Wi-Fi reduces RF design/certification risk; enough Flash/RAM for Wi-Fi stack, TLS, OTA, and our control logic; deterministic MCU timing for PWM/LED fade; lower power than an SBC; straightforward migration from our Python logic to C/FreeRTOS or Harmony tasks.

- Cons: Less headroom than Linux for heavyweight analytics at the edge; requires a C/RTOS port of our prototype.

C. NXP i.MX RT (e.g., RT1060) + IW416 Wi-Fi

- What it offers: 600 MHz Cortex-M7 “crossover MCU” with large on-chip SRAM (~1 MB) and external QSPI Flash; Wi-Fi via IW416 (802.11 a/b/g/n + BT 5) module; extensive SDKs and RTOS options. (Raspberry Pi)

- Pros: Very high performance MCU with rich peripherals; scalable to more advanced UI (e.g., TFT) later; strong vendor support.

- Cons: Two-chip solution (MCU + Wi-Fi) increases integration and certification complexity vs. a single Wi-Fi MCU module; cost/power likely higher than WFI32 for a simple thermostat.

Quick Comparison

| Requirement | Raspberry Pi | Microchip WFI32E01 | NXP i.MX RT + IW416 |
| --- | --- | --- | --- |
| Wi-Fi | Integrated (Broadcom) (Raspberry Pi) | Integrated 802.11 b/g/n (single module) (w11stop.com) | External IW416 (802.11 a/b/g/n + BT 5) (Rajguru Electronics) |
| Flash/RAM | GB-class RAM on SBC | Up to 2 MB Flash / 512 KB RAM (w11stop.com) | ~1 MB SRAM on-chip + external QSPI Flash typical (Raspberry Pi) |
| Peripherals | GPIO, I²C, UART, (software) PWM | GPIO, PWM timers, I²C, UARTs | GPIO, PWM, I²C, UARTs |
| Power/Cost | Highest | Lowest (single RF-certified module) | Medium (two-chip) |
| Certification | SBC route; Wi-Fi pre-cert may not cover end product | Module with pre-cert options | RF module + host certification |
| Fit for thermostat | Great for prototype | Best fit for production | Strong, but more integration work |

Recommendation

Adopt Microchip WFI32E01 for production. It meets all peripherals (PWM/I²C/UART/GPIO), has integrated Wi-Fi, and provides sufficient Flash/RAM for control, secure TLS, and OTA. It minimizes RF risk and BOM while keeping our control timing precise on an MCU.

Cloud Integration Plan (Phase 2)

- Network & Security: Use WPA2/WPA3 client; TLS 1.2+ with hardware-assisted RNG; provision device certificates at manufacture. (Supported by Microchip’s Harmony + WolfSSL stack on WFI32E01.) (w11stop.com)

- Protocol: MQTT over TLS to SysTec’s broker; JSON payload: { ts, state, tempF, setpointF, humidity, rssi }.

- Update/Device Management: Secure OTA using signed images; fail-safe A/B partitions where available.

- Data cadence: Keep UART CSV (for service) and publish MQTT every 30–60 s configurable.

- Observability: Add a debug topic and on-device ring buffer exposed via a “support” command.

Mapping Prototype → Production

| Prototype Feature | Production (WFI32E01) |
| --- | --- |
| AHT20 on I²C | Same I²C driver; use low-power sleep between samples |
| 16×2 LCD (HD44780 4-bit) | Drive via GPIO with microsecond-level timing (Harmony delay or timer) |
| PWM LED fade/solid | Use timer/PWM peripherals; same thresholds as prototype |
| Buttons (mode/up/down) | GPIO interrupts with debounce (software) |
| UART CSV (every 30 s) | Keep for service logs; primary telemetry moves to MQTT/TLS |
| State machine (OFF/HEAT/COOL) | Implement as RTOS task with event queue (button, timer, sample) |

Risks & Mitigations

- Wi-Fi robustness: Use connection watchdog; exponential back-off; store unsent samples in NVM.

- Security/OTA: Enforce signed firmware and secure boot chain; rotate credentials.

- Supply chain: Select a module with longevity program; second-source LCD/AHT20 footprints.

- Thermal control accuracy: Add hysteresis (e.g., ±0.5 °F) and minimum on/off times in firmware.

References

Microchip Technology. WFI32E01 Module (PIC32MZ-W1) overview & specs. https://www.microchip.com/en-us/product/WFI32E01 (Integrated 802.11 b/g/n, up to 2 MB Flash, 512 KB RAM, MCU-class peripherals). (w11stop.com)

NXP Semiconductors. i.MX RT1060 Crossover MCU family summary. https://www.nxp.com/products/processors-and-microcontrollers/arm-microcontrollers/i-mx-rt-crossover-mcus/i-mx-rt1060-crossover-mcu-family-1mb-on-chip-ram:NXP-IMXRT1060 (600 MHz Cortex-M7 with large on-chip SRAM, external QSPI Flash). (Raspberry Pi)

NXP Semiconductors. IW416 Wi-Fi + Bluetooth 5 solution. https://www.nxp.com/products/wireless/wi-fi-plus-bluetooth/2-4-5-ghz-dual-band-wi-fi-4-iw416-wi-fi-plus-bluetooth-5-combo-solution:IW416 (802.11 a/b/g/n + BT 5; typical pairing with i.MX RT). (Rajguru Electronics)

Raspberry Pi. Raspberry Pi 3 Model B product page (on-board wireless LAN/BLE; 40-pin GPIO). https://www.raspberrypi.com/products/raspberry-pi-3-model-b/ (Raspberry Pi)

SparkFun. Raspberry Pi 3 Model B+ overview (dual-band 802.11ac, 1 GB RAM). https://learn.sparkfun.com/tutorials/raspberry-pi-3-b/specifications (useful spec recap for the B+ variant used widely for prototyping). (الکترو ولت)