Interfaces compared: GPIO, PWM, and UART

What they are (and how they differ)

- GPIO: a single digital pin I can configure at runtime as input or output. In output mode it drives a logic high/low; in input mode it reads high/low (optionally with pull-ups/downs). There is no timing or messaging protocol—just discrete logic levels I control directly in software. Throughput is limited by how fast the OS and my code can toggle/read pins and by electrical constraints.

- PWM: a digital pin electrically, but driven by a periodic waveform where I set the frequency and the duty cycle (percentage of time high in each period). Hardware PWM engines (or timed software) keep the waveform running with minimal CPU involvement. PWM conveys analog-like intent (e.g., brightness, motor speed) by varying duty cycle, not by sending bytes/frames.

- UART: true serial communications interface that moves bytes using a framed format (start bit, 8/7 data bits, optional parity, stop bit) at an agreed baud rate. It uses two data wires (TX and RX; plus ground) and is asynchronous (no clock wire). Once configured, the hardware shifts bits in/out; my software reads/writes byte buffers.

Why I would choose one over another:

Choose GPIO when I need simple on/off control or sensing with minimal overhead:

- Turn an LED fully on/off, read a pushbutton, latch a chip-select line, bit-bang a very simple custom handshake, etc.

- Pros: trivial to use, no protocol required, immediate response.

- Cons: doesn’t encode multi-bit data by itself; software-timed toggling can be jittery under Linux.

Choose PWM when I need a steady, time-accurate proportion of “on” vs. “off”:

- Dim an LED smoothly, set a fan/motor speed, drive a servo (with the right frequency/period).

- Pros: analog-like control with a single pin; hardware PWM gives stable timing and low CPU.

- Cons: it’s not a data link—great for control signals, not for sending bytes or messages.

Choose UART when I need to exchange bytes reliably between devices with minimal wiring:

- Talk to a GPS, a BLE module, an MCU, or a USB-to-TTL adapter.

- Pros: simple protocol, huge ecosystem, buffering handled in hardware/driver.

- Cons: point-to-point (not multi-drop without extra transceivers), fixed baud/format must match, limited speed compared to SPI/I²C.

Practical tradeoffs I keep in mind

- Signal semantics: GPIO = discrete states, PWM = continuous control via duty cycle, UART = byte stream.

- Timing: PWM and UART rely on hardware timing; GPIO timing depends on my software and the OS scheduler.

- Wiring: GPIO/PWM = one data pin each (plus ground). UART = two data pins (TX/RX) plus ground.

- CPU load: PWM/UART hardware frees the CPU; rapid GPIO bit-banging can consume CPU and suffer jitter.

Use cases we just implemented:

- PWM on GPIO 18 to fade an LED.

- UART via /dev/ttyUSB0↔/dev/ttyS0 to pass text commands between a USB-serial adapter and the Pi, then toggle a GPIO to actuate the LED.

I use GPIO for binary control/sense, PWM when I need stable, analog-like control with a single digital pin, and UART when I need a simple, robust byte stream interface between devices.