Lab Questions:

- Why do SerialTest-Write.py and SerialLightControl-Client.py use encode() when writing?

Serial ports transmit bytes, not Python Unicode strings. In Python three, str is Unicode; serial.Serial.write() expects a bytes-like object. Calling encode() (typically UTF-8) converts the command text (e.g., "ON\r\n") into a byte sequence the UART can actually send.

- Why does SerialTest-Read.py use decode() when reading?

pySerial’s read()/readline() return bytes. To print, compare, or parse that data as text, you convert bytes back to a Python str with decode() using the same character encoding you used on transmit (commonly UTF-8). This mirrors the writer’s encode() step so text round-trips correctly.

- What is the purpose of the try/except blocks in the client and server?

Graceful termination: Catch KeyboardInterrupt (Ctrl-C) to avoid an abrupt crash and to run cleanup code (e.g., close the serial port, stop loops, report a tidy message). Robustness to I/O faults: Serial links can drop or misbehave (unplugged cable, framing error, garbage input). Wrapping I/O and command handling in try/except lets the program log/report the issue, recover or exit cleanly, and still perform finalizers (e.g., GPIO reset).

- Why must GPIO pins be returned to their original state at the end?

Leaving pins configured as outputs or left driven HIGH can cause:

- Unexpected current draw (LEDs half-on; external circuits energized),

- Interference with later programs that assume default (input) pin modes,

- Undefined board state across runs.

Calling the library’s cleanup (e.g., GPIO.cleanup() in RPi.GPIO) releases resources and resets pins to safe defaults, ensuring repeatable behavior for the next execution.

References

Liechti, C. (n.d.). pySerial API. In pySerial documentation. https://pyserial.readthedocs.io/en/latest/pyserial_api.html

Python Software Foundation. (n.d.). Unicode HOWTO. In Python 3 documentation. https://docs.python.org/3/howto/unicode.html

Python Software Foundation. (n.d.). Errors and exceptions. In The Python tutorial (Python 3 documentation). https://docs.python.org/3/tutorial/errors.html

RPi.GPIO Project. (n.d.). Basic usage. https://sourceforge.net/p/raspberry-gpio-python/wiki/BasicUsage/